# PART 2

(L3SW Intervlan) Static Route ,WAN, STP Portfast, Priv Exec Sec, and Port Aggregation

This project is a small improvement on the previous simple Inter-VLAN lab without WAN connectivity. All commands required to recreate this project are included in this document.

In this lab, I used several shortcut commands that helped me complete multiple tasks more efficiently and enter configurations more quickly without ambiguity.

If you have any questions regarding subnetting or configuration, please feel free to email me at aeroncharlies@gmail.com.

128	64	32	16	8	4	2	1

Given network: 192.168.52.0 /x - MAKATI

Network A / VLAN 9 | CS – 25 Hosts

2^n-2 = usable hosts, n = number of host bits

2^5-2 = 30 usable hosts

To find subnetmask, we subtract 32 to the number of host bits. 32 – 5 = 27

/27 = 255.255.255.224

Network Address: 192.168.52.0 255.255.255.224

First Usable Address: 192.168.52.1 255.255.255.224

Last Usable Address: 192.168.52.30 255.255.255.224

Broadcast Address: 192.168.52.31 255.255.255.224

Next Subnet: 192.168.52.32

Network B / VLAN 14 | IT – 10 Hosts

2^n-2 = usable hosts, n = number of host bits

2^4-2 = 14 usable hosts

To find subnetmask, we subtract 32 to the number of host bits. 32 – 4 = 28

/28 = 255.255.255.240

Network Address: 192.168.52.32 255.255.255.240

First Usable Address: 192.168.52.33 255.255.255.240

Last Usable Address: 192.168.52.46 255.255.255.240

Broadcast Address: 192.168.52.47 255.255.255.240

Next Subnet: 192.168.52.48

Given network: 192.168.54.0 /x – LAGUNA

Network C / VLAN 30 | Sales – 60 Hosts

2^6-2 = 62 usable hosts

/26 = 255.255.255.192

Network Address: 192.168.54.0 255.255.255.192

First Usable Address: 192.168.54.1 255.255.255.192

Last Usable Address: 192.168.54.62 255.255.255.192

Broadcast Address: 192.168.54.63 255.255.255.192

Next Subnet: 192.168.54.64

Network D / VLAN 20 | HR – 32 Hosts

2^6-2 = 62 usable hosts

/26 = 255.255.255.192

Network Address: 192.168.54.64 255.255.255.192

First Usable Address: 192.168.54.65 255.255.255.192

Last Usable Address: 192.168.54.126 255.255.255.192

Broadcast Address: 192.168.54.127 255.255.255.192

Next Subnet: 192.168.54.128

Network E / VLAN 14 | IT – 27 Hosts

2^5-2 = 30 usable hosts

/27 = 255.255.255.224

Network Address: 192.168.54.128 255.255.255.224

First Usable Address: 192.168.54.129 255.255.255.224

Last Usable Address: 192.168.54.158 255.255.255.224

Broadcast Address: 192.168.54.159 255.255.255.224

Next Subnet: 192.168.54.160

WAN A / L3SW100 <> R100 |  2 Hosts

2^2-2 = 2 usable hosts

/30 = 255.255.255.252

Network Address: 192.168.54.160 255.255.255.252

First Usable Address: 192.168.54.161 255.255.255.252

Last Usable Address: 192.168.54.162 255.255.255.252

Broadcast Address: 192.168.54.163 255.255.255.252

Next Subnet: 192.168.54.164

Note1: Pwede naman /31 para sa point-to-point connection, pero /30 na ang gamitin.

Note2: 2 offices are using ethernet copper cable only as a sample. Sa totoong practice, ang gamit na dyan ay fiber optic dahil sa layo ng location.

WAN B / R200 <> R100 | 2 Hosts

2^2-2 = 2 usable hosts

/30 = 255.255.255.252

Network Address: 192.168.54.164 255.255.255.252

First Usable Address: 192.168.54.165 255.255.255.252

Last Usable Address: 192.168.54.166 255.255.255.252

Broadcast Address: 192.168.54.167 255.255.255.252

Next Subnet: 192.168.54.168

Configuration

Hostname Configuration

SW400

Switch>en

Switch#conf t

Switch(config)#host SW400

SW300

Switch>en

Switch#conf t

Switch(config)#host SW300

MSW500

Switch>en

Switch#conf t

Switch(config)#host MSW500

R100

Router>en

Router#conf t

Router(config)#host R100

Access Ports Configuration on Switches w/ VLAN creation/enrollment

SW400

SW400>en

SW400#conf t

Enter configuration commands, one per line. End with CNTL/Z.

SW400(config)#int ran f0/2-3

SW400(config-if-range)#sw mo ac

SW400(config-if-range)#sw acc vlan 14

% Access VLAN does not exist. Creating vlan 14

SW400(config-if-range)#int f0/1

SW400(config-if)#sw mo ac

SW400(config-if)#sw ac vlan 30

% Access VLAN does not exist. Creating vlan 30

SW400(config-if)#exit

SW400(config)#vlan 14

SW400(config-vlan)#name IT

SW400(config-vlan)#vlan 30

SW400(config-vlan)#name SALES

SW300

SW300>en

SW300#conf t

SW300(config)#int ran f0/1-2

SW300(config-if-range)#sw mo ac

SW300(config-if-range)#sw ac vlan 14

% Access VLAN does not exist. Creating vlan 14

SW300(config-if-range)#int f0/3

SW300(config-if)#sw mo ac

SW300(config-if)#sw ac vlan 20

% Access VLAN does not exist. Creating vlan 20

SW300(config-if)#int ran f0/4-6

SW300(config-if-range)#sw mo ac

SW300(config-if-range)#sw ac vlan 30

% Access VLAN does not exist. Creating vlan 30

SW300(config-if-range)#exit

SW300(config)#vlan 14

SW300(config-vlan)#name IT

SW300(config-vlan)#vlan 20

SW300(config-vlan)#name HR

SW300(config-vlan)#vlan 30

SW300(config-vlan)#name SALES

Trunk Port Configuration on Switches SW300 and SW400

SW300

SW300>en

SW300#conf t

SW300(config)#int f0/7

SW300(config-if)#sw mo tru

SW300(config-if)#sw tru allowed vlan 14,30

SW400

SW400#conf t

SW400(config)#int f0/7

SW400(config-if)#sw mo tru

SW400(config-if)#sw trunk allowed vlan 14,30

Port Aggregation on SW300 <> MSW500 link

SW300

SW300>en

SW300#conf t

SW300(config)#int ran g0/1-2

SW300(config-if-range)#channel-group 1 mode desirable

MSW500

MSW500>en

MSW500#conf t

MSW500(config)#int ran g1/3-4

MSW500(config-if-range)#channel-group 1 mode desirable

Trunk Port Configuration on Switches SW300 and MSW500

SW300

SW300(config)#int p1

SW300(config-if)#sw mo tru

SW300(config-if)#sw trunk allowed vlan 14,20,30

MSW500

MSW500>en

MSW500#conf t

MSW500(config)#int ran g1/3-4

MSW500(config-if-range)#sw mo tru

MSW500(config-if-range)#sw trunk allowed vlan 14,20,30

Inter-VLAN via SVI on MSW500

MSW500

MSW500>en

MSW500#conf t

Enter configuration commands, one per line. End with CNTL/Z.

MSW500(config)#int vlan 14

MSW500(config-if)#ip add 192.168.54.158 255.255.255.224

MSW500(config-if)#int vlan 20

MSW500(config-if)#ip add 192.168.54.126 255.255.255.192

MSW500(config-if)#int vlan 30

MSW500(config-if)#ip add 192.168.54.62 255.255.255.192

Routed Port Config on MSW500 <> R100

MSW500

MSW500>en

MSW500#conf t

Enter configuration commands, one per line. End with CNTL/Z.

MSW500(config)#int g1/5

MSW500(config-if)#no switchport

MSW500(config-if)#ip add 192.168.54.161 255.255.255.252

MSW500(config-if)#no shut

R100

R100>en

R100#conf t

Enter configuration commands, one per line. End with CNTL/Z.

R100(config)#int g0/1

R100(config-if)#ip add 192.168.54.162 255.255.255.252

R100(config-if)#no shut

Connecting R200 <> R100

R100

R100>en

R100#conf t

R100(config)#int g0/0

R100(config-if)#ip add 192.168.54.166 255.255.255.252

R100(config-if)#no shut

R200

R200>en

R200#conf t

Enter configuration commands, one per line. End with CNTL/Z.

R200(config)#int g0/0

R200(config-if)#ip add 192.168.54.165 255.255.255.252

R200(config-if)#no shut

Creating Static Routes to connect MAKATI branch <> LAGUNA branch

MSW500

MSW500>en

MSW500#conf t

MSW500(config)#ip route 192.168.52.0 255.255.255.224 192.168.54.162

MSW500(config)#ip route 192.168.52.32 255.255.255.240 192.168.54.162

R100

R100>en

R100#conf t

R100(config)#ip route 192.168.52.0 255.255.255.224 192.168.54.165

R100(config)#ip route 192.168.52.32 255.255.255.240 192.168.54.165

R100(config)#ip route 192.168.54.128 255.255.255.224 192.168.54.161

R100(config)#ip route 192.168.54.64 255.255.255.192 192.168.54.161

R100(config)#ip route 192.168.54.0 255.255.255.192 192.168.54.161

R200

R200>en

R200#conf t

R200(config)#ip route 192.168.54.128 255.255.255.224 192.168.54.166

R200(config)#ip route 192.168.54.64 255.255.255.192 192.168.54.166

R200(config)#ip route 192.168.54.0 255.255.255.192 192.168.54.166

STP Portfast Configuration on all end hosts

SW100

SW100>en

SW100#conf t

SW100(config)#int ran f0/1-3, f0/5

SW100(config-if-range)#span port

SW200

SW200>en

SW200#conf t

SW200(config)#int ran f0/1-2, f0/3

SW200(config-if-range)#span portf

SW300

SW300>en

SW300#conf t

SW300(config)#int ran f0/1-6

SW300(config-if-range)#spann portfa

SW400

SW400>en

SW400#conf t

SW400(config)#int ran f0/1-3

SW400(config-if-range)#span por

Privilege Exec mode protection (type 5 hashing)

SW100

SW100>en

SW100#conf t

SW100(config)#enable secret hulaanmo

SW200

SW200>en

SW200#conf t

SW200(config)#enable secret hulaanmo

SW300

SW300>en

SW300#conf t

SW300(config)#ena sec hulaanmo

SW400

SW400>en

SW400#conf t

SW400(config)#ena sec hulaanmo

R200

R200>en

R200#conf t

R200(config)#ena sec hulaanmo

R100

R100>en

R100#conf t

R100(config)#ena sec hulaanmo

MSW500

MSW500>en

MSW500#conf t

MSW500(config)#ena sec hulaanmo

//naglagay na ko kaunting security para naman may line of defense yung config dun sa device.